总体思路: 模型计算辐照度考虑光照: 直射+散射(包括反射)

---- 1.光伏组件直射入射角的计算

---- 2.结合pvlib库得到南极某地区理论法向直射辐照度与散射辐照度

---- 3.年理论辐照度

1.光伏组件直射入射角的计算

->单面直射入射角计算:

4角度融合: 太阳俯仰角 太阳方位角 光伏板俯仰角 光伏板方位角

cos(入射角) = cos(太阳俯仰角 - 光伏板俯仰角) * cos(太阳方位角 - 光伏板方位角)

考虑多面围合的组合方位角计算(assume第一个面方位角是0):

第i个面:

cos(入射角i) = cos(太阳俯仰角 - 光伏板俯仰角) * cos(太阳方位角 - (方位角+360/n * i))

2.结合pvlib库得到南极某地区理论法向直射辐照度与散射辐照度

pvlib库计算直射辐照度 和 给出某地某时刻DNI的方法:

-> 根据天文学理论推导得到某时刻某地的太阳俯仰角和太阳方位角

标准时间 -->  儒略日和儒略世纪数 --> 太阳黄经 太阳赤纬 --> 太阳俯仰角 太阳方位角

-> 计算某地考虑部分大气与环境因素的DNI - Direct Normal Irradiance(法向直射辐照度):

I : 太阳常数(理论计算式如下, T为太阳表面温度, r为太阳半径, R为地日距离, 实际取值为近地轨道空间站实际测量的单位面积法相直射辐照度)

m：大气质量

k：与大气浑浊度、吸收和散射有关的经验系数（由 Linke turbidity 给出）

-> 考虑散射辐照度(地面散射+天空散射)

计算DHI (Diffuse Horizontal Irradiance)

其中GHI(Global Horizontal Irradiance) 为实测得地表总辐照度(包括直射+散射), pvlib库会根据输入得经纬度调出当地或就近地区测得的GHI数据.

为太阳高度角

我的模型中天空散射直接取DHI

地面散射(Gound Diffusion):

地面散射本质是由GHI反射得到, 所以大致为GHI 乘一个系数, 这里大致

用GD =

----------------------------------------------------------------------------------------------------------

** Optimized_GD(t)=GHI(t)⋅α(t)⋅0.98⋅(1−cos(β))/2​

其中: α(t)=kdiff​(t)⋅αws​(t)+(1−kdiff​(t))⋅αbs​(θSun​,t)

在晴天（kdiff​ 较小），随角度变化的 αbs​ 占据主导。

在阴天（kdiff​ 较大），恒定的 αws​ 权重更高。

这里使用MODIS卫星的数据, 输入太阳天顶角, 得到实时变化的ρ值, 优化掉定值0.85

其中ρ为albedo反照率, 南极地区大致取0.85

β 为光伏板倾角(正面与地面夹角)

* SIback 为雪地反射光谱与硅基电池光谱失配的光谱修正系数, 根据研究取0.98 (&1)

* 利用 MODIS 和 Sentinel-2 卫星数据 结合可以获得更高精度的反照率时间序列

α=kdiff​⋅αws​+(1−kdiff​)⋅αbs​(θSun​)

αws​ (White-sky albedo)：假设天空是各向同性散射时的反照率。

αbs​(θSun​) (Black-sky albedo)：随太阳高度角变化的直射光反照率。这正是修正非各向同性的关键项。

卫星数据不仅考虑了各向同性（在处理散射光时），更重要的是它修正了非各向同性（在处理直射光时），使得在低太阳高度角的南极地区，计算出的反射辐照度更接近真实情况。

获取数据： 下载南极目标区域的历史 MODIS BRDF 参数 (MCD43A1)。

从 pvlib 获取当前的太阳天顶角 (θSun​)。

-----------------------------------------------------------------------------------------------------------------------

-> 得到某一时刻某个面总辐照度TI(t) = DNI(t) ·cos(入射角(t)) + DHI(t) + GD(t)

----------------------------------------------------------------------------------------------------------------------------

***

引入入射角修正因子 (IAM)

单面光伏板表面的玻璃层在南极这种高纬度、低太阳高度角的环境下，会产生显著的反射损失。

当光线以大角度入射时（南极常态），实际进入电池片的辐照度会远低于理论值。

在计算直射辐照项时，引入 Sandia 入射角修正模型 或 Martin and Ruiz 模型。

TI(t)=DNI(t)*cos(AOI)*fIAM(AOI)+DHI(t)*fIAM,diff*((1+cosβ)/2)+ 	GHI(t)*αeff′(t)*((1-cosβ)/2)*fIAM,diff​​

其中:αeff′(t)=[kdiff(t)*αws(t)+(1−kdiff(t))*αbs(θSun,t)]*0.98*flow_sun(γ)

以Perez模型为基础

->e.g. Gsky​=DHI⋅(Fiso​+Fcircumsolar​+Fhorizon​)

poa= pvlib.irradiance.get_total_irradiance( surface_tilt=beta, surface_azimuth=gamma, dni=dni, ghi=ghi, dhi=dhi, solar_zenith=theta_z, solar_azimuth=azimuth, albedo=, model='perez' )

G_beam = poa['poa_direct']
G_sky = poa['poa_sky_diffuse']
G_ground = poa['poa_ground_diffuse']

已经做的5个优化:

- FlAM

为了修正南极低太阳高度角下，光伏板玻璃表面的角度反射损失，本模型引入了入射角修正因子 (fIAM)。其中，直射项修正参考 Sandia IAM 模型，而天空散射项的修正因子 (fIAM,diff​) 则参考 Martin and Ruiz 分析模型，以确保在大入射角条件下有效利用辐照度计算的准确性。

IAM(θ)=1−b0​(1/cosθ​−1)

-> ($1 2.4) Angular reflection losses at the front and rear module surface are accounted for with the Sandia incident angle modifier (IAM) model (King et al., 2004). An IAM profile for a c-Si module with non-antireflective coated glass was used for the rear side.

->($2 2.2&3.1)  One problem of the reference cell, however, is that reflections from its planar glass cover significantly reduce the measured effective irradiance at high angles of incidence. This effect can be modelled approximately using incident angle modifier (IAM) models. In Fig. 2 we show the result of fitting the 1-parameter Martin and Ruiz IAM model [23] to the reference cellangularresponseusingaparameterar=0.163. This enables an angular correction to be applied, albeit with the introduction of signif-icant uncertainty. We discuss the implications of the angular and spec-tral effects further in section 3.1.

In the first, we modify the reference cell albedometer measurement with an angular-response correction to reference cell irradiance measurements: where G’sky and G’ground are the incident-angle corrected irradiances measured, derived from the sky-facing and ground-facing reference cell measured irradiances Gsky and Gground, respectively. kdiff is the diffuse light fraction, fIAM(θ) is the incident angle modifier (IAM) function for direct light, and fIAM,diff is the IAM for diffuse light. We used the Martin and Ruiz [23] IAM model with fit parameter ar = 0.163 and fIAM,diff = 0.986 derived by assuming isotropic diffuse radiance. As θSun and kdiff are not measured directly, they were inferred from calculation [29] and estimation [30] respectively, using location, time and GHI as inputs. This method is relatively complex to implement and relies on estimation and assumptions.

- 2.0.98 光谱错配

- ->($1 3.3 fig12) The spectral nature of albedo is shown to have significant influence on bifacial photo-voltaic performance wherein backside spectral impact as high as 1.20 is observed for fixed-tilt systems above green vegetation and as low as 0.98 for systems above snow.

- αeff′(t)

kdiff(t)*αws(t)+(1−kdiff(t))*αbs(θSun,t), 核心在于将到达地面的太阳辐射拆分为“直射”与“散射”两个物理分量，并分别考虑地表对这两种不同光性质的反射特性。αws为white sky albedo, while αbs为black sky albedo, 这两个由BRDF卫星数据获得.

- ->($1 2.3) In principle, the calculation of albedo from BRDF requires the 5- dimensional integration of the product of spectrally and angularly resolved incident radiance with the spectrally resolved BRDF. This is complex and requires more information than is typically available. For simplicity, it is convenient to separate into two components: White-sky albedo αws and black-sky albedo αbs, representing diffuse and direct illumination components respectively. αws is calculated from a bihemispherical integral of the surface BRDF assuming an isotropic light source (diffuse sky). αbs is calculated from a single hemispherical integral of the BRDF assuming that all incoming light is from a single direction (the Sun). αbs is a function of solar zenith angle θSun, whereas αws is independent of the Sun’s position. The total albedo α estimated from a weighted sum of these sources:

- α = kdiff αws + ( 1 − kdiff ) αbs

- where kdiff is the quotient of diffuse horizontal irradiance (DHI) divided by global horizontal irradiance (GHI). We used this approach to estimate effective albedo using the following procedure

4.flow_sun(γ)

- 当太阳从 10° 降向地平线时，太阳光在大气中穿行的路径极长。这导致水平面散射辐照度与水平面直射辐照度的比例（DHI/GHI）迅速增加。

- BRDF 模型是基于经验核函数的数学拟合。当太阳处于极低角度时，这些核函数会进入数学上的不稳定区，导致计算出的 αbs数值可能出现物理上不可能的极值（如大于 1.0 或剧烈震荡

- The daily horizontal diffuse to global fraction (Kd) on the clear days was less than 0.40, while on cloudy days Kd was greater than 0.95. Albedo increases with decreases in sun height on clear days, but only down to solar elevation angles of about 10◦ at which point the albedo decreases (Iqbal, 1983). This phenomenon occurs because the fraction of horizontal diffuse to horizontal beam irradiance rapidly increases as the solar elevation angle decreases from 10◦ toward the horizon. Therefore, we only show measurements when the sun is higher than 10◦ above the horizon in Fig. 4a, Fig. 4c, and Fig. 4e. (他没有记10°以下的数据, 说明需要修正)

此模型还需做4个优化:

---> 雪损模型（Snow Loss Model）

**[论文3]: 单面组件在严寒环境下冬季雪损平均达 33.12%。

落地逻辑（Marion 模型）：

判定滑动： 当 Tambient​>GPOA​/(−80) 时，雪开始滑动。

单面板劣势： 单面板没有背面加热效应，雪滑得比双面板慢得多。

雪深影响： 来源 5 发现，当雪深超过 3cm 时，由于地面干扰（Ground Interference），滑动系数 sc 会从 0.4 锐减至 0.06。

--->电气失配与安装排布（Portrait vs. Landscape）

模型目前计算的是“面”的辐照度，但实际发电取决于“组串”的电流。

阴影特征：积雪通常从顶部下滑并堆积在组件底部。

排布影响：

纵向（Portrait）： 即使只有底部 10% 的雪，也会切断整个组串电流，导致 100% 功率损失。

横向（Landscape）： 配合旁路二极管，可以保留上方未遮挡区域的电力。

对 DW 的优化意义： 如果最优β较小，必须在模型中加入一个“布局折减因子”,或者强制建议在南极采用横向排布。

--->阵列自遮挡（Ground Self-Shading）

[论文2] 光伏阵列会在地面投射长阴影。

GD 项假设地面是全亮的，但在南极极低太阳角下，组件前面的地面很大一部分处于阵列自身的阴影中。

需要对 αeff’(t) 项进行几何阴影折减。如果光伏板“看到”的地面 50% 是黑的，那么反射项的贡献也要减半。

--->双面或许更优

------------------------------------------------------------------------------------------------------------------------------

- 年理论辐照度

-> 将以下5个数据输入pvlib, pvlib由上述方法得到该地一整年的理论DNI GHI 太阳方位角 太阳俯仰角数据:

纬度  latitude

经度  longitude

时区  timeZone         -->  DNI  GHI  太阳方位角  太阳俯仰角

年份  year

反照度  albedo=0.85

在这之后, 应当能够建立一个

input: latitude, longtitude, timeZone, year, albedo(ρ)

total faces, tilt angle

output: total addressable irradiation for every sepcific moment in the year(TI(t))

的模型A

输入的光伏组件模型是一个n个面围成的, 每个面倾角都是β的几何图形.

每半个小时取一个点, 每个点通过模型A计算一次, 得到的TI(t) x 1800(s), 加总得到全年的理论辐照度.

这个辐照度数值本身没有太大的参考价值, 因为没有考虑的环境因素, 积雪, 光伏板的其他特性等, 不过足以满足通过对比选出最优的n和β的目的, 针对此目的无需考虑哪些没有考虑的因素.

模型成果: 可以得到变量n β变化下全年理论总辐照度的变化趋势, 通过分析综合其他因素选出最优的n β, 以此模型为基础进行阴影遮挡的组件阵列的计算和建模.

最优的组合面数 (n) 和 倾角 (β)。

阴影遮挡计算  组件阵列建模

雪地的光谱反照率在300-1100nm非常高且平坦，但由于雪地反射光发生了“蓝移”，导致硅基电池的光谱影响系数（SI）约为 0.98

**Supplement: 雪损(单双面):

单面光伏板在严寒多雪环境下，其积雪损失远高于双面组件。

论文3: 单面组件在冬季的平均雪损高达 33%，而双面组件仅为 16%。这是因为单面组件缺乏背面吸收热量导致的“自加热除雪效应”，积雪融化和滑动极慢。

引入 Marion 雪损模型，并针对南极调整滑动系数 (sc)。

根据环境温度（Tamb​>0∘C 或特定阈值）判定积雪是否开始滑动。

单面组件的滑动系数应设定得更小（来源 5 建议在某些屋顶结构下小至 0.06），以反映除雪难的问题

($1)

include green vegetation, dry vegetation, gravel, and snow. Spectral mismatch and spectral impact are calculated for the front and backside of three different bifacial cell concepts mounted on horizontal single axis trackers and fixed-tilt substructures. The spectral nature of albedo is shown to have significant influence on bifacial photo-voltaic performance wherein backside spectral impact as high as 1.20 is observed for fixed-tilt systems above green vegetation and as low as 0.98 for systems above snow. The results reveal that spectral impact is always lower on tracked than fixed-tilt systems because a greater fraction of sky diffuse light reaches the backside of tracked systems. Given the variety of albedos tested here, we find that the normalized difference vegetation index is a good predictor of backside spectral effects. When the high-resolution measurements are truncated to 4 to 8 carefully selected wavelengths, we find that this limited measurement resolution sufficiently captures the sea-sonal spectral albedo fluctuations that influence bifacial photovoltaic energy production. Finally, to alleviate the dearth of spectral datasets presently available to the PV community, the spectral irradiance and albedo mea-surements are made freely available in open access format